TRACK DAY BIKE PREP

1. CHECK FOR FLUID LEAKS:

Engine oil

Forks fluid

Rear Shock

Brake fluid

Coolant (if applicable)

2. TIRE WEAR:

If worn 50% or more, they will fail Tech     Inspection. Look at wear bar in tread (The wear bar is way past 50%). If any doubt, you should just replace them.

3. INSPECT BRAKES AND PADS :

If brake pads are worn 50% or more, they will fail Tech Inspection. If any doubt, you should replace them. Also you should check hydraulics, make sure no leaks and then apply the brake very firmly both front and rear.

4. LIGHT AND LENSES :

Always check with the Track Day Tech Inspectors before the Track Day. Every Track Day/ School is different from one company to another. Fishtail does not require the removal of lenses or glass, but they require tape over glass and lenses. Some Track Day companies require duct tape or painters tape.

You also need to remove the mirrors and deactivated the brake light.

Note: Most track day/school companies have instructions on their website and have Tech Inspectors helping in the morning before the track day begins.

5. RUNNING CONDITION :

Make sure the bike is in good running condition. Throttle returns freely and bike doesn’t smoke or leak fluid.

6. CHAIN AND SPROKETS :

Chain is properly lubed / adjusted and sprockets are in good condition.